| # Contaminant | Medium | Holder | Flow rate | Analysis |
| --- | --- | --- | --- | --- |
| Total inhalabale dust | Glass fibre filter (or membrane filter) | 7 hole or IOM sampling head | 2 litres/minute | Gravimetric |
| Metal s and metal compounds (dusts) | Membrane filter | 7 hole or IOM sampling head | 2 litres/minute | Atomic absorption spectroscopy (AA) or Inductively coupled plasma – atomic emission spectroscopy (ICP-AES) |
| Lead and lead compounds | Membrane filter | 7 hole or IOM sampling head or 1 hole lead head | 2 litres/minute 1 litre/minute | Atomic absorption spectroscopy (AA) or Inductively coupled plasma – atomic emission spectroscopy (ICP-AES) |
| Respirable dust | Glass fibre filter (or membrane filter) | Cyclone sampling head | 2.2 litres/minute (must be used to ensure respirable fraction collected) | Gravimetric |
| Crystalline silica | Silver membrane filter | Cyclone sampling head | 2.2 litres/minute (must be used to ensure respirable fraction collected) | X-ray diffraction Or Infra-red spectroscopy |
| Asbestos | Gridded membrane filter | Cowled open face head | 1 to 8 litrs/minute depending on objective of survey and sampling period | Filter put on microscope slide ans cleared using acetone vapour and triacetin. Then examined under microscope |
| Fibre glass | Sampling can be undertaken for total inhalable dust (method as above) or for respirable fibres using same equipment as for asbestos. |  |  |  |

| # Contaminant | Medium | Holder | Flow rate | Analysis |
| --- | --- | --- | --- | --- |
| Organic vapours | Carbon (activated charcoal) Porous polymer (e.g. Tennax or Chromosorb) | Tube Tube | 50 to 200 mls/minute 50 to 200 mls/minute | Solvent (typically carbon disulphide) desportion followed by gas chrpomatography with flame ionisation detector (GC-FID) Thermal desportion followed by gas chrpomatography with flame ionisation detector Note GC-FID can only be used if the compounds sampled are known. For identification of unknown compounds mass spectrometry is normally used instead of the FID |
| Organic vapours (polar compounds) | Silica gel | Tube | 50 to 200 mls/minute | Solvent desportion followed by gas chrpomatography with flame ionisation detector |
| Inorganic gases (typical method) | Depneds on the gas being sampled. However, traditionally a liquid medium was used | Midget impinger or bubbler used with liquid media | Typically 1 litre/minute | “wet chemistry” – often colorimetric analysis |
| Simple method for organic and inorganic gases and vapours | Colorimetric indicator tubes (e.g. Draeger tubes). Not particularly accurate and are usually “grab samples” (i.e. short period static samples rather than personal samples over a full shift). However, they can give a good initial indication on whether there is a problem or not. |  |  |  |