| Contaminant | Effects | Sources | Monitoring Parameter | Control Options |
| --- | --- | --- | --- | --- |
| Organic matter (biodegradable) | Microoganisms consume organic matter leading to reduction in dissolved oxygen | Sewage, animal wastes, some industrial effluents, run off from soil | BOD COD |  |
| Plant nutrients (nitrates & phosphates) | Excesive plant growth & algal blooms reduces oxygen level and can cut out light | Fertilisers Animal wastes Detergents Some industrial effluents | Nitrates Phosphates |  |
| Suspended solids | Reduction in light penetration (increased turbidity) | Some industrial processes (e.g. paper mills, quarries) Soil erosion | Turbidity Suspended solids |  |
| Acids / alkalis | Many organisms can only tolerate limited pH range Can dissolve metals | Industrial processes (e.g. plating, chemical manufacture) Some rocks | pH |  |
| Toxic chemicals | Various effects on flora and fauna. Can affect humans | Industrial processes | Various - depends on chemicals |  |
| Oils (and other immiscible liquids) | Reduced dissolved oxygen Unpleasant | Industrial processes Traffic Motor related Domestic waste | Oil content |  |
| Trace organics | Odour Affects taste | Industrial processes | Various - depends on chemicals |  |
| Endocrine disrupers | Feminisation of fish | Chemical industry Intensive farming Sewage |  |  |
| Heat | Affects breeding / growth Can alter ecology | Industrial processes (e.g. power plants, glass works, steel works) | Temperature |  |
| Micro-organisms | Pathogenic | Animal excrement |  |  |